___________ __________ _____________________ Roll No Section Student Signature

Digital Logic Design (EE1005)

Date: April 9th 2026

Course Instructors:

Dr. Rabia Tabassum, Sir Javed Quershi, Dr. M Adeel, Ms. Hira Tunio, Ms. Sana Naz, Mr. Ishtiaq Ahmed, Mr. M Miqdad Khan, Mr. M Rahim, Mr. Abuzar Zafar, and Dr. Muhammad Nouman Durrani

| Sessional-II Exam |  |
| --- | --- |
| Total Time (Hrs): | 1 |
| Total Marks: | 30 |
| Total Questions: | 2 |

Do not write below this line

Attempt all the questions in the given sequence.

CLO # 2: Demonstrate the acquired knowledge to apply techniques related to the design and analysis of digital electronics circuits, including Boolean Algebra and Multi-variable  Karnaugh map methods.

Design and analysis of digital electronics circuits.				[ 6 + 2 = 8 Marks]

Time: 20 Minutes

Q1: (a) Consider a 3-bit unsigned binary number X=X2​X1​X0​. A logical circuit produces an output Z=Z2​Z1​Z0​

such that:

Z = X−1

For example:

- If X=000, then Z=111 		 ► Z = 000 + (2’s complement of 1)  	 ► Z= 000+111

- If X=101, then Z=100		 ► Z = 101 + (2’s complement of 1)  	 ► Z= 101+111

- Construct the complete truth table for the above 3-bit logical circuit.

- Derive the simplified Boolean expressions for each output bit Z2​,Z1​,Z0​ using K-map minimization.

- Draw the logical circuit of the above Boolean expression for Z2 only using NAND gate.

(b)  Implement the NOT gate using a 2:1 Multiplexer.

CLO # 3 Analyze   small –scale combinational digital circuits.

Design and analysis of Combinational Logic Circuits: 	    	[4 + 4 + 4 + 2 + 2 + 6 = 22 Marks]

Time: 40 Minutes

Q2: (a) Using only four-bit adders, construct an eight-bit adder. Each four-bit adder has two four-bit inputs and one five-bit output. Your eight-bit adder should have two eight-bit inputs and a one eight-bit output.

(b) Using two 2-to-4 decoders with enable (EN) inputs, construct a 3-to-8 decoder. Provide a labeled logic diagram

and explain how the enable signals are utilized.

(c)  Implement the given Boolean function using an 8-to-1 multiplexer (8×1 MUX). Assume A₀ as the least

significant bit (LSB).

F( A3, A2 , A1 , A0 )  = m (0, 3, 4, 5, 7, 11, 13,14)

(d)  A 7-segment decoder/driver drives the display in the given Figure 1. If the waveforms are applied as indicated,

determine the sequence of digits that appears on the display. [Solve here on the question paper].

Figure 1

Start 0, 1, 6, 9, 4, 4, 4, 8, 0

(e)	The serial data-input waveform (Data in) and data-select inputs (S0 and S1) are shown in Figure 2. Determine the data-output waveforms on D0 through D3 for the demultiplexer in Figure 2. [Solve here on the question paper].

Figure 2

Do	1       0	      0	 0        0       0       0       0

D1	0       1        0      0        0       0       0       0

D2	0       0        1      0        0       0       1      0

D3     	0       0        1      1        0       0       0      1

(f) 	A 4-to-2 encoder is a combinational circuit that converts one active input out of four into a 2-bit binary code.

Now consider a 4-to-2 encoder with inputs D3,D2,D1,D0 ​ and outputs Y1​,Y0​. Assume that only one input is HIGH at a time.

(i) Construct the truth table of the encoder.

(ii) Derive the Boolean expressions for outputs Y1​ and Y0​.

- What will happen if more than one input is HIGH simultaneously? Briefly explain.

If more than one input is HIGH at the same time:

- The output becomes ambiguous (incorrect)

- Example:

  - If D2​=1 and D3​=1

  - Output becomes:

Y1​=1,Y0​=1

  - This corresponds to D3​, but D2​ is also active → information loss

A simple encoder cannot handle multiple active inputs.
This limitation is resolved using a priority encoder.